# Contribution from the EDM group to be included in the
SPRINT@NICA Conceptual Design Report
Search for the electric dipole moment of light nuclei at the NICA complex

V. V. Fimushkin, A. V. Butenko, E. A. Butenko, N. V. Dunin, K. A. Ivshin, M. V. Kulikov, S. A. Kostromin, V. P. Ladygin, V. E. Laryonov, E. M. Syresin, A. N. Solovev, I. S. Volkov, V. A. Lebedev, E. E. Donets, V. V. Bleko, N. M. Piskunov, A. Ya. Silenko, O. V. Teryaev, Yu. N. Uzikov, A. B. Borisov, E. F. Dlin (JINR)

Yu. N. Filatov, A. N. Zelenski, S. V. Vinogradov, S. N. Zhabin, I. S. Yudin, A. A. Chernikova, E. D. Tsyplakov, I. V. Lilienberg, A. I. Chernyshov (MIPT)

A. E. Aksentiev, A. S. Belov, Yu. V. Senichev, S. D. Kolokolchikov, A. A. Melnikov, P. Palamarchuka (INR RAS)

N. N. Nikolaev (ITP)

A. M. Kondratenko, M. A. Kondratenko (STL Zaryad)

V. G. Baryshevsky, S. V. Anischenko, A. A. Gurinovich (INP BSU)

D. K. Toporkov (BINP SB RAS)

# Contents

1 Introduction

2 Lattice design principles for a storage-ring EDM search

2.1 Spin motion in electromagnetic fields

2.2 Frozen spin concept

2.3 Quasi-frozen spin concept

2.4 Parameters of the Wien filters

3 NICA collider with bypasses

3.1 Modified NICA lattice for the EDM search

3.2 Lattice parameters

3.3 Bypass optics

3.4 Natural chromaticity and dynamic aperture

3.5 Spin-orbit motion with optimized Wien filters

3.6 Betatron chromaticity and spin coherence

3.7 Summary

4 Modernized Nuclotron

4.1 Present Nuclotron lattice

4.2 Modernization options

4.3 Nuclotron 2.1: quasi-frozen spin for deuterons

4.4 Nuclotron 2.2: quasi-frozen spin with Wien filters for deuterons

4.5 Nuclotron 2.2.2: quasi-frozen spin with Wien filters for protons

4.6 Maximum energy of heavy ions and protons in Nuclotron 2.2.2

4.7 Features of the modernized Nuclotron lattice

4.8 Summary

5 Specialized frozen spin rings at JINR

5.1 Frozen spin option for the proton beam

5.2 Frozen spin option for the deuteron beam

5.3 Summary

6 Transition energy crossing for polarized protons

7 Spinor technique for spin-orbit tracking

8 Electrostatic elements

9 MDM systematics and tensor polarizability

10 Spin precession in the axion search mode

11 Conclusion

Acknowledgments

References

# 1 Introduction

An experiment to search for the electric dipole moment (EDM) at the NICA collider requires an alternative spin-control method based on the “quasi-frozen spin” (QFS) concept. In contrast to the “frozen spin” method proposed at BNL, which requires the construction of a specialized storage ring [1], the quasi-frozen spin method is applicable to virtually any accelerator, including rings that were not originally designed for EDM searches. In this method, the spin does not keep a fixed orientation along the direction of motion over the entire revolution. Instead, it deviates from the momentum in an arc with magnetic dipoles and recovers its original orientation in a straight section. This is achieved with elements combining electric and magnetic fields, known as Wien filters, installed in the straight section: the spin rotation accumulated in the arc is compensated by the opposite rotation in the Wien filters. The fields can be chosen so that the net Lorentz force vanishes and the closed orbit is not perturbed, which allows the Wien filters to be installed in straight sections. Consequently, a polarimeter placed downstream of the Wien filters always observes the same orientation of the spin vector, i.e., from its point of view the spin is “frozen”.

For this purpose, it is proposed to install bypass sections in the NICA collider to house the Wien filters. Apart from the EDM physics goals, there are two reasons for this modification of the lattice. First, the existing straight sections do not have enough free space for the Wien filters. Second, the present lattice is optimized for the collider mode of operation. Therefore, the bypass channels create an alternative straight section parallel to the original one. The NICA collider can then operate as a storage ring for EDM experiments with polarized deuterons in the quasi-frozen spin mode. Beam dynamics studies of the designed bypass channel confirm the feasibility of this approach.

The second facility considered as a candidate for EDM experiments is the modernized Nuclotron. The aim is to define the relationship between the Nuclotron and the NICA collider and to modify the Nuclotron lattice so as to obtain a dual-function accelerator: a booster of polarized proton and deuteron beams for injection into the collider and an independent storage ring for precision EDM experiments with light nuclei. The design follows these conditions:

- the modernized ring must fit into the existing Nuclotron tunnel;

- the proton and deuteron polarization must be preserved and controlled throughout the NICA complex;

- the polarized proton beam must not cross the transition energy in the collider after electron cooling, so that the design luminosity can be reached;

- a frozen or quasi-frozen spin mode must be implemented either in the collider or in the Nuclotron, depending on which ring is more suitable for EDM research.

The developed lattice satisfies all these conditions and provides a basis for implementing this option.

The third option is a specialized ring housed in a separate building. Two lattices of a specialized storage ring for EDM searches with polarized proton and deuteron beams have been developed. In contrast to the NICA and Nuclotron options, which rely on the quasi-frozen spin mode, the specialized rings are based on the frozen spin concept. Owing to the different magnitude and sign of the proton and deuteron magnetic anomalies, the two lattices differ substantially: the proton beam requires a purely electrostatic ring with a total length of about 400 m, whereas the deuteron beam requires a ring with combined E+B deflectors and a total length of about 150 m.

Thus, three options are currently available for EDM research with light charged nuclei at the NICA complex [2]. For all of them, the spin coherence time of the beam must reach about 1000 s [3].

# 2 Lattice design principles for a storage-ring EDM search

## 2.1 Spin motion in electromagnetic fields

The concept of an EDM measurement in a storage ring is based on the Thomas–Bargmann–Michel–Telegdi (T-BMT) equation [4,5]. In accordance with the Ehrenfest theorem, it describes the classical spin motion of a charged particle with a hypothetical EDM:

dSdt&=Ωmdm+Ωedm×S,Ωmdm&=−emγ(γG+1)B⟂+(1+G)B∥−γG+γγ+1β×Ec,Ωedm&=−eη2mβ×B+Ec,  G=g−22,  (1)

where γ is the Lorentz factor, β is the relative velocity, c is the speed of light, e and m are the particle charge and mass, G is the magnetic anomaly, g is the gyromagnetic ratio, Ωmdm and Ωedm are the spin precession frequencies due to the magnetic and electric dipole moments, η is a dimensionless coefficient defined by d=ηeℏ/4mc, and B={B⟂,B∥} and E={E⟂,E∥} are the magnetic and electric fields. Since no elements with a longitudinal magnetic field are used, we set B∥=0; the longitudinal electric field is neglected as well, E∥=0, because its contribution is small.

The lattice must provide conditions under which the EDM signal continuously accumulates while the beam circulates in the ring. To this end, the spin and the longitudinal momentum must remain aligned, S↑↑p, with a precision that determines the attainable sensitivity of the EDM measurement. Depending on the precision and the method of its realization, this condition is referred to as the frozen spin or, more generally, the quasi-frozen spin condition [6].

## 2.2 Frozen spin concept

The idea of searching for the proton and deuteron EDMs with polarized beams in a storage ring is based on the frozen spin (FS) method originally proposed at the Brookhaven National Laboratory [1,7]. An FS lattice consists of deflectors combining electric and magnetic fields in one element, in which the spin of the reference particle always stays along the momentum.

The frozen spin method relies on the fact that at a specific, so-called “magic” energy the spin precesses in the external fields with the same frequency as the particle momentum, ΩE,Bp=eEmγβc+eBmγ. Subtracting this frequency from Ωmdm gives the spin precession frequency relative to the momentum:

Ωmdmp=ωEp+ωBp,  (2)

where ωEp=emG−1γ2−1E×βc and ωBp=emG B⟂ are the contributions of the electric and magnetic fields. In a purely electrostatic ring (B=0) the frozen spin condition ωEp=0 is satisfied at the magic energy

G−1γmag2−1=0.  (3)

In a ring with both magnetic and electric elements, the frozen spin condition Ωmdmp=0 is achieved by balancing the radial electric field Er against the vertical guiding magnetic field Bv [1]:

Er=GBvcβγ21−Gβ2γ2≈GBvcβγ2.  (4)

A hybrid ring is required for deuterons because their magnetic anomaly is negative, G=−0.143, and condition (3) cannot be satisfied.

It is convenient to introduce the spin tune, i.e., the number of spin precessions per revolution. In an electrostatic ring the spin tune relative to the momentum is νsE=ωEp/ΩEp:

νsE=G−1γ2−1γβ2,  (5)

and in a magnetic field, νsB=ωBp/ΩBp:

νsB=γG.  (6)

Thus, the same concept applies to protons and deuterons, but it is realized with different types of deflectors.

## 2.3 Quasi-frozen spin concept

In the NICA ring, an implementation of the FS concept would require a complete rebuild of the optics. Suppose instead that the spin oscillates in the horizontal plane around the frozen spin direction with a small amplitude, Φs2≪1. This can be done with special electric-field elements in the straight sections, which bring the spin back along the momentum after it has deviated in the magnetic arc. The EDM signal growth is then reduced by the factor J0(Φs)≈1−Φs2/4. The deuteron magnetic anomaly G=−0.143 is small, and the spin oscillates around the momentum within half of the spin phase advance in the magnetic arc, Φs=πγG/2≈0.25. Therefore, at the optimal parameters of the EDM measurement, γ=1.12, the effective EDM signal is reduced by only a few percent.

This gives the essence of the quasi-frozen spin concept [6]: the spin is not frozen relative to the momentum but continuously oscillates around an average fixed direction that coincides with the momentum direction. The question to be answered next is how to implement a variable MDM spin precession in the storage ring, which is needed for its zero average over the statistics, while providing a sufficient growth of the EDM signal.

Two QFS options exist. In the first one, the magnetic and electric fields are completely separated in space: the magnetic field acts in the arcs and the electric field acts in the straight sections, which are realized as negative-curvature “reverse” electrostatic arcs. This option inherits the drawback of cylindrical electrodes, namely, a whole set of high-order field nonlinearities. In the second option, a small magnetic field of about 100 mT is added to the spin-aligning E+B elements; it compensates the Lorentz force of the electric field and makes the elements straight. Such an E+B element is a Wien filter. The second option is the most appropriate one for rings not designed specifically for the EDM search, in particular for the NICA ring with bypass sections, whereas for the Nuclotron, where the free space is limited, the first option is the baseline.

## 2.4 Parameters of the Wien filters

In a magnetic arc the momentum is rotated by the angle ΦarcB, and the spin simultaneously rotates in the horizontal plane relative to the momentum by the angle

Φsarc=γG ΦarcB.  (7)

In the E+B elements of the straight section, the spin rotation relative to the momentum has two components. In the electric field it is opposite to the momentum rotation,

ΦsE=−γG+γγ+1β2 ΦssE,  (8)

where ΦssE is the momentum rotation angle in the electric field. In the magnetic field the spin rotates in the same direction as the momentum, ΦsB=(γG+1) ΦssB, where ΦssB is the momentum rotation angle in the magnetic field. Since the Lorentz force vanishes, the two momentum rotation angles are equal, ΦssE=ΦssB, and both spin rotations can be expressed through one of them, e.g., through ΦssB=eBssmγvLss, where Bss and Lss are the magnetic field and the length of the straight element. The QFS condition requires that the net spin rotation in the E+B elements compensate the spin rotation in the arc, ΦsB+ΦsE=Φsarc. For an arc bending the beam by ΦarcB=π this gives

(γG+1) ΦssB−γG+γγ+1β2 ΦssE=πγG.  (9)

Simple transformations yield the parameters of the E+B elements:

LΣEss=GG+1 mc2e πβ2γ3,  Bss=−Esscβ,  (10)

where LΣ is the total length of the straight elements in one straight section. For a realistic electric field of 100 kV/cm, the corresponding magnetic field is below 96 mT. This simplifies the technical design: a permanent magnet or an air-core coil can be used.

The material of this section has been reported in [6,8–11].

# 3 NICA collider with bypasses

## 3.1 Modified NICA lattice for the EDM search

The NICA lattice was not designed for EDM research; hence the only option is the quasi-frozen spin concept, which can be realized for deuterons. The adaptation strategy relies on two facts. First, the straight sections of the collider are fully occupied by the equipment of the MPD and SPD experiments. Second, these detectors and their interaction points are not needed for the EDM search. The simplest solution is therefore to introduce bypasses, as shown in Fig. 1.

Figure 1. Modified NICA lattice for the EDM search with bypass insertions (red).

The bypass sections avoid both detector interaction points and provide free space for E+B elements. Owing to the zero Lorentz force, these elements are transparent to the beam dynamics, while they rotate the spin opposite to its rotation in the arcs and thus realize the quasi-frozen spin concept. Without the interaction points, the two beams circulate in two uncoupled rings, which doubles the data-taking rate. Figures 2 and 3 show the Twiss functions and the dynamic aperture in both transverse planes.

Figure 2. Twiss functions of NICA with bypass insertions.

(a); (b).

Figure 3. Dynamic aperture of NICA with bypass insertions in the horizontal (a) and vertical (b) planes.

The linear part of the dynamic aperture, about 2000 mm·mrad, significantly exceeds the required value in both planes. For an electric field limited to about 100 kV/cm, Eq. (10) gives a total length of the E+B elements of about 25–30 m per bypass, which is 30% of the bypass length. Spin-orbit tracking shows that the spin deviates from the momentum by less than 10°; the simulation for the three spin projections gives a deviation within ±7.5∘.

[Note: The source document gives both values (10° and ±7.5∘) in two versions of this paragraph; to be unified.]

Despite this deviation, the polarization asymmetry measured at the polarimeter location remains constant.

## 3.2 Lattice parameters

The polarimetry constraints [12] determine the choice of the particle species and the experiment energy. The lattice is designed for deuterons with a kinetic energy of 240 MeV. The corresponding dipole field and magnetic rigidity are listed in Table 1.

Table 1. Main parameters of the experiment and the bypass lattice.

| Particles | deuterons |
| --- | --- |
| Dipole magnetic field | 0.132 T |
| Magnetic rigidity Bρ | 3.252 T·m |
| Ring circumference | 503.04 m |
| Experiment energy | 240 MeV |

## 3.3 Bypass optics

The arc geometry of the NICA ring is kept unchanged; only the fields in the installed elements may be changed. NICA can therefore be used for different experiments without a major rebuild of the lattice.

The NICA arc has a non-zero dispersion, and both the dispersion and its derivative vanish at the arc ends. The straight sections are dispersion-free along their whole length. The circumference of the original NICA ring is Lacc=503.04 m and the length of one arc is Larc=142.15 m, which leaves (Lacc−2Larc)/2=109.6 m for each straight section. The bypass channels contain an alternative straight section and sections with bending dipoles. The dipoles deflect the beam by α=9∘; their field is BBP=1 T and their length is LdipBP=50 cm. The alternative straight section is displaced by 1 m from the original one, so the length of the bypass channel is LBP=1 m/sinα≈6.4 m. The general layout of the bypass channels is shown in Fig. 1.

The bending magnets perturb the dispersion function. At least two focusing quadrupoles in the bypass channel are therefore needed to suppress the dispersion at its exit and to keep the whole straight section dispersion-free. One or three defocusing quadrupoles can be used to keep the beta functions periodic and symmetric. Two cases with adapted straight sections, identical to the arcs but without dipoles, are considered first; this simplifies the modeling of an ideal regular lattice. Finally, a realistic lattice with a fully regular FODO straight section is considered.

#### Bypass with three quadrupoles.

In this case the bypass channel contains the minimum of three quadrupoles: two focusing quadrupoles QBP1 and one defocusing quadrupole QBP2 (Fig. 4). The arc is matched to the bypass channel by three quadrupoles QM1, QM2 and QM3 (matching section M1), and the bypass is matched to the straight section by the same quadrupoles (M2). This is possible because of the symmetry between the arc and the straight section built into the original lattice. The section lengths are given in Fig. 4 and Table 2. The ring circumference becomes L3quadacc=503.46 m.

Figure 4. Layout of the bypass with three quadrupoles.

Figure 5 shows the Twiss functions; the black lines mark the channel boundaries. The maximum of βy is located at the center of the channel and can be large compared with βx. For this reason, a bypass with five quadrupoles is also considered.

Figure 5. Twiss functions of the bypass with three quadrupoles. The black lines show the positions of the bypass bending magnets.

#### Bypass with five quadrupoles.

Here the bypass channel contains five quadrupoles of two families, focusing QBP1 and defocusing QBP2. The channel becomes longer, L5quadBP=9.35 m, and the displacement increases to 1.46 m (Fig. 6). The matching sections M1 and M2 are still identical but consist of two quadrupoles, QM1 and QM2, which keep the Twiss functions regular. The circumference increases to L5quadacc=510.02 m. As Fig. 7 shows, the maximum of βy at the channel center becomes smaller, whereas the maximum of the dispersion grows from Dx3quad∼0.2 m to Dx5quad∼0.5 m. This case therefore has to be adapted to the real lattice.

Figure 6. Layout of the bypass with five quadrupoles.

Figure 7. Twiss functions of the bypass with five quadrupoles. The black lines show the positions of the bypass bending magnets.

#### Realistic lattice.

Based on these two cases, a lattice as close as possible to the real machine has been designed. It has a fully regular straight section, which becomes shorter, LSSBP=80.71 m (Fig. 8). The bypass channel contains five quadrupoles and displaces the alternative straight section by 1.46 m from the original one. Different matching sections M1 and M2 are used to compensate the asymmetry between the arc and the straight section. The Twiss functions of one half of the NICA ring with bypasses are shown in Fig. 9; the Wien filters are placed at the center of the straight section. All Twiss functions were computed with OptiM [13]. The lengths and tunes of all versions are summarized in Table 2.

Figure 8. Layout of the bypass in the realistic lattice.

Figure 9. Twiss functions of one half of the NICA ring with bypasses. The Wien filters are located in the straight section.

Table 2. Ring circumference, lengths of the individual sections and betatron tunes Qx/Qy of the lattice versions.

| Lattice | Circumference, m | Straight section, m | Bypass, m | Matching section, m | Qx/Qy |
| --- | --- | --- | --- | --- | --- |
| NICA (ion mode) | 503.04 | 109.6 | – | – | 9.44/9.44 |
| 3 quadrupoles | 503.46 | 83.97 | 6.41 | 3.198 | 13.8/11.8 |
| 5 quadrupoles | 510.0 | 83.97 | 9.35 | 2.548 | 13.44/11.44 |
| Realistic | 503.5 | 80.70 | 9.35 | 2.548 | 12.8/11.8 |

## 3.4 Natural chromaticity and dynamic aperture

This subsection addresses the stability of the orbital motion, namely, the correction of the natural chromaticity of the NICA bypass lattice. The chromaticity is the dependence of the betatron tune Q on the momentum deviation δ [14]:

Q′=dQdδ.  (11)

The natural chromaticity is produced by the quadrupoles at locations with non-zero beta function:

∂Qx∂δ=−14π0Lβx(s) kx(s) ds,  (12)

where βx(s) is the beta function at the quadrupole and kx(s) is the quadrupole gradient. The natural chromaticity can be corrected with sextupoles:

∂Qx∂δ=14π0Lβx(s) r(s) D(s) ds,  (13)

where D(s) is the dispersion function and r(s) is the sextupole gradient. Correction is possible only if the sextupoles are placed where both the dispersion and the beta function are non-zero.

Since the arc lattice is regular, the beta and dispersion functions are regular along the arc as well, except at the arc ends. There the dispersion is suppressed with the missing-magnet method, which breaks the regularity of the Twiss functions (Fig. 9). Two essentially different sextupole families can thus be placed in the regular part of the arc: (1) at the maximum of D and βx and the minimum of βy, and (2) at the minimum of D and βx and the maximum of βy.

Figure 10 shows the betatron tune versus the energy deviation for the natural chromaticity and for the chromaticity corrected with two sextupole families. Figure 11 shows the dynamic aperture without sextupoles and with the chromaticity corrected. The tracking was performed with COSY Infinity [15]. The main parameters are listed in Table 3.

Table 3. Chromaticity of the NICA ring with bypasses.

| Natural chromaticity ξx/ξy | −17/−17 |
| --- | --- |
| Corrected chromaticity ξx/ξy | −7×10−6/7×10−6 |
| Number of sextupole families | 2 |

Figure 10. Betatron tune shift versus the energy deviation in the horizontal (left) and vertical (right) planes. Red: natural chromaticity; blue: corrected chromaticity.

(a); (b). (c); (d).

Figure 11. Dynamic aperture of the NICA ring with bypasses: (a), (b) natural chromaticity without sextupoles; (c), (d) chromaticity corrected with two sextupole families. Left: horizontal plane; right: vertical plane.

## 3.5 Spin-orbit motion with optimized Wien filters

The modification of the lattice is intended primarily for EDM research. This implies that the ring must operate in the quasi-frozen spin mode and that the spin coherence time of the beam must be long enough. The first requirement is met by placing Wien filters in the straight sections; the standard way to meet the second one is to use several sextupole families that correct the betatron and spin chromaticities. Both points were studied numerically with COSY Infinity [15].

In the standard formalism, the spin transfer matrix is an SO(3) rotation T(ν,n) parametrized by the normalized spin precession frequency (spin tune) ν and the invariant spin axis n. If the quasi-frozen spin condition holds, T is the identity matrix and ν=0. In the proposed lattice this is achieved by tuning the electric field of the E+B elements, while the magnetic field is chosen so as to keep the closed orbit unchanged. The simulation shows that the spin tune of the reference particle ν0 can indeed be set to zero (Fig. 12). The spin deviates by a small angle in the arc and recovers its orientation in the Wien filters of the straight section (Fig. 13).

Figure 12. Spin tune ν0 of the reference particle versus the electric field of the E+B elements.

Figure 13. Spin components Sx, Sy, Sz of the reference particle along one half of the ring (arc and straight section).

The spin chromaticity is the dependence of the spin precession frequency on the particle orbit length. Because of it, the spins of the beam particles spread around the invariant spin axis n. This process, called spin decoherence, is irrelevant for experiments with a non-zero expected polarization along n. In EDM experiments, however, the polarization component orthogonal to n is of interest; its expectation value is zero, hence the importance of the spin coherence time (SCT).

The spin chromaticity of the beam can be compensated with sextupole families to a level that provides a sufficiently long spin coherence time (Fig. 14). Classically, three sextupole families placed at the maxima of βx, βy and D should be used, but the lattice under study does not allow an efficient separation of all three Twiss functions. It was also found that the betatron and spin chromaticities cannot be compensated simultaneously in this combined lattice; this problem is discussed in the next subsection.

(a); (b).

Figure 14. Spin components versus the number of turns for particles with different initial offsets, without (a) and with (b) sextupole families.

## 3.6 Betatron chromaticity and spin coherence

According to the theory of Ref. [16], the main cause of spin decoherence is the deviation of the particle energy from the equilibrium value:

Δδeq=γs2γs2α0−1δm22α1+32βs2γs2−α0γs2+1γs4+ΔLLβ,  (14)

ΔLLβ=π2Lϵxνx+ϵyνy.  (15)

The energy level around which the particle oscillates is called the effective energy. To eliminate the effect of the momentum spread on the orbit lengthening completely, the factor κ=η1=α1+32βs2γs2−α0γs2+1γs4 in Eq. (14) must vanish. In addition, the orbit lengthening due to the betatron motion must be compensated as well. Both are achieved with three independent sextupole families placed at the maxima of the beta and dispersion functions. With sextupoles in the lattice, Eq. (15) takes the more general form [17]

ΔLLβ=−πLϵxξx+ϵyξy.  (16)

#### Magnetic ring.

To study this effect, a purely magnetic ring without electric elements is considered first; similar calculations were carried out for the COSY ring in Jülich [17]. This ring is a version of NICA in which the arcs are unchanged and the straight sections are regular FODO cells matched to the arcs with two quadrupoles. The sensitivity of the lattice to the sextupole strengths rf and rd is considered. The sextupole layout is shown in Fig. 15; here the families SF1 and SF2 are identical and form one focusing family.

Figure 15. NICA collider arc with the sextupole layout.

ξx&=−13.302+4522.234 rf+527.934 rd,ξy&=−13.337−944.735 rf−2214.642 rd,α1&=0.037−16.617 rf−2.537 rd.  (17)

Two sextupole families cannot optimize all three parameters of the spin-orbit motion. Either the pair (ξx,ξy) or the pair (ξy,α1) can be tuned. The second choice is probably less practical, because α1 changes only weakly and the energy contribution to the spin tune spread can be reduced by beam cooling.

In a purely magnetic lattice, the motion in the x–δ plane is fully described by the theory. In the vertical plane, spin resonances may contribute to the spread of the spin precession frequencies.

As Figs. 16(a)–(c) show, the optimum of the spin chromaticity does not coincide with that of the betatron chromaticity. Without optimization, Δνs∼10−8 at frev∼1 MHz gives an SCT of about 100 s, which is insufficient for the EDM experiment. The vertical chromaticity must therefore be optimized while the dynamic aperture is preserved. A change of κ by about ±1 is not decisive for the SCT.

(a); (b); (c).

Figure 16. Purely magnetic ring: spin tune spread (a) versus the horizontal chromaticity for particles injected with different radial offsets; (b) versus κ=η1 for particles with an energy offset; (c) versus the vertical chromaticity for particles with a vertical offset.

As noted above, spin resonances produce an additional spread of the spin precession frequencies Δνs in the vertical plane. At the working point γG=0.16, the nearest resonance is the intrinsic betatron resonance γG=10−Qy=0.2. For a resonance strength ε=10−4 and a detuning δ=0.04, the expected spread due to the resonance is

Δνs=δ−δ2+ε2∼1.2×10−7.  (18)

This value estimates Δνs at ξy=0 in Fig. 16(c). Increasing the vertical chromaticity reduces the spread of the spin precession frequencies through the betatron orbit lengthening. A simple estimate for a particle with εy∼1 mm⋅mrad and ξy=40 gives

Δνs=νsΔLLβ=−γGπL εyξy=−4×10−8,  (19)

which is what is needed to suppress the typical spin tune spread at ξy=0 in Fig. 16(c).

#### Lattice with Wien filters.

As Figs. 17(a)–(c) show, in a ring with electrostatic fields the optima of the betatron and spin chromaticities do not coincide either in the x–δ plane or in the vertical plane. As in the magnetic ring, the preferred option for minimizing spin decoherence is to compensate the pair (ξx,ξy) for the beam parameters given above. The residual spread Δνs∼10−9 in the energy plane gives an SCT of about 1000 s, which is sufficient for the EDM experiment. Moreover, measurements of (ξx,ξy) are routine in storage rings, unlike measurements of α1; with two sextupole families it is therefore reasonable to optimize the pair (ξx,ξy).

(a); (b); (c).

Figure 17. Ring with Wien filters: spin tune spread (a) versus the horizontal chromaticity for particles injected with different radial offsets; (b) versus κ=η1 for particles with an energy offset; (c) versus the vertical chromaticity for particles with a vertical offset.

The mismatch between the optima of the betatron and spin chromaticities in the x–δ plane of a ring with electrostatic fields is explained by the particle motion in the electric field. A radial displacement of a particle in a deflector with a radial field changes the particle energy. This follows from the potential nature of the field and couples the orbital motion to the particle energy.

#### NICA with bypasses.

For the modified NICA ring with bypasses and Wien filters described above, the betatron chromaticity and the spin coherence were optimized separately.

To correct the betatron chromaticity, two sextupole families are needed, as shown above: one near the focusing quadrupoles and the other near the defocusing quadrupoles. The natural chromaticity of the NICA ring with bypasses is ξx,y=−17/−17. After the correction, the spin precession frequency can be followed in Fig. 18: the red line shows the natural chromaticity and the blue dotted line shows the chromaticity corrected to zero. For this case, spin tracking over 3×105 turns was performed for particles with different initial offsets in x, y and δ and with the initial spin S0 at 45° in the y–z plane (Fig. 14).

The key quantity is the spin precession frequency, which in general depends on the coordinates and the energy. The dominant contribution is the quadratic term in the expansion of the spin precession frequency, as seen in Fig. 18 for both the natural and the corrected chromaticity.

To obtain spin coherence, the quadratic terms of the expansion were compensated with two focusing sextupole families. These families, however, are linearly dependent, because they are located at positions with equal values of the beta and dispersion functions (Fig. 15), which is typical of lattices with regular arcs. COSY Infinity cannot operate near a zero spin precession frequency: the spin transfer matrix becomes degenerate near the resonance and the code fails. The lattice was therefore detuned from the resonance to νs∼10−4, while the particles were still required to precess synchronously, i.e., coherently. In this approach the chromaticity is not controlled; its value is only monitored, and with the spin chromaticity corrected it equals ξx,y=−13/−18.

Correcting only the betatron chromaticity does not give a zero spread of the spin precession frequency. Conversely, achieving spin coherence by suppressing the quadratic term of the spin precession frequency does not correct the chromaticity. This brings us back to Eq. (14): to ensure a zero energy deviation, the chromaticities ξx, ξy and η1 must all be set to zero. This is possible with three sextupole families, but it still does not yield spin coherence. Moreover, the required maximum sextupole strength is too high to be realized (Table 4).

This effect has already been explained for the purely magnetic ring, where the dependence of the spin tune on the sextupole strengths is given by Eq. (17). In the presence of electrostatic fields, the chromaticities ξx, ξy affect the spin tune more strongly than κ, whose contribution can be compensated by beam cooling.

(a); (b); (c).

Figure 18. Spin precession frequency versus the coordinates x (a), y (b) and δ (c) for different optimization cases: NC, natural chromaticity (red); BC, zero betatron chromaticity (blue dotted); SC, spin coherence (green); BC​α, zero chromaticity and α1=0 (violet); BC​η, zero chromaticity and η1=0 (light blue).

Table 4. Main parameters of the NICA ring with bypasses for different optimization cases. Kx, Ky, Kz are the coefficients of the quadratic terms of the spin precession frequency in x, y and δ.

| Optimization | None | Chromaticity | Spin coherence | Chromaticity + η1 |
| --- | --- | --- | --- | --- |
| Chromaticity ξx/ξy | −17/−17 | 0/0 | −13/−18 | 0/0 |
| α1 | 0.2 | −0.4 | −0.37×10−2 | −0.85 |
| Kx | −0.16×10−1 | 0.55×10−1 | 0.27×10−13 | 0.56×10−1 |
| Ky | 0.51×10−2 | 0.76×10−1 | −0.12×10−12 | 0.78×10−1 |
| Kz | −0.43×10−1 | 0.20×10−1 | 0.13×10−12 | 1.6×10−1 |
| Sextupole families | none | 2 | 3 | 3 |
| Max. sextupole strength, m​−3 | – | 2.7 | 19.4 | 104.2 |

## 3.7 Summary

For EDM experiments, NICA must operate as a storage ring. To this end, a modification is proposed in which bypass channels create alternative straight sections parallel to the original ones. These straight sections accommodate Wien filters that compensate the MDM spin rotation in the arcs. Since the arcs remain unchanged, NICA can still be used for other experiments.

Two basic bypass layouts were considered, and finally the most realistic lattice with a fully regular straight section was obtained. The final design meets all lattice requirements. Spin-orbit simulations with optimized Wien filter fields show that the spin recovers its orientation in the straight section, so that the quasi-frozen spin method can be implemented at NICA with bypasses. The simultaneous control of the spin coherence and the betatron chromaticity is under study. The same lattice also makes it possible to use NICA as an axion antenna (Section 10).

The material of this section has been reported in [8,11,16,18–24].

# 4 Modernized Nuclotron

## 4.1 Present Nuclotron lattice

Figure 19 shows the NICA accelerator complex, which comprises the Booster, the Nuclotron and the Collider. The Booster and the Nuclotron serve as the injector of heavy ions, polarized protons and deuterons into the collider. The Nuclotron lattice was originally designed to be as close to circular as possible and has eight superperiods. Figure 20 shows the Twiss functions of one superperiod, which consists of a bending arc of three FODO cells, each with four bending magnets with a maximum field of 1.8 T, and a straight section of one FODO cell. The Nuclotron circumference is 251 m.

Figure 19. NICA accelerator complex.

Figure 20. Twiss functions of one Nuclotron superperiod.

The main drawback of the Nuclotron, both as a collider injector and as a stand-alone ring, is the limited length of the drift spaces in which the equipment for these two functions must be installed. All modernization options below are assessed with respect to this limitation.

## 4.2 Modernization options

Two modernization options of the Nuclotron have been studied.

The first option, Nuclotron 2.1, is a lattice that preserves all properties of the existing accelerator and allows deuteron EDM experiments with electric deflectors or Wien filters installed in the ring. Either device keeps the spin of the polarized beam along its direction of motion around the ring. Within the eight-superperiod lattice, Nsuper=8, this requires implementing the frozen (or quasi-frozen) spin concept by lengthening the straight sections between the arcs, ensuring zero dispersion in the straight sections, and, if possible, keeping the ring circumference so that the equipment fits into the existing tunnel. An axion search could also be included in this framework, although the treatment of the systematic errors of such an experiment is not yet fully understood. In the Nuclotron 2.1 option, all key collider parameters, such as the transition energy and the electron cooler parameters, remain unchanged. This creates a serious problem at the interface between the collider and the Nuclotron: crossing the transition energy with a cooled beam leads to irreversible beam degradation and a lower final luminosity.

The Nuclotron 2.2 option introduces modifications that ensure compatibility with the collider. The injection energy of polarized protons into the collider remains at 2–3 GeV. To eliminate the effect of transition crossing after electron cooling, the transition energy of the collider is to be raised above its maximum operating energy. In addition, a lower maximum energy of heavy ions and polarized particles in Nuclotron 2.2 is considered. This frees space in the ring for Wien filters and extends the capabilities of the Nuclotron for EDM and axion experiments. A lower Nuclotron energy does not degrade the complex as a whole, since the collider can take over some functions of the Nuclotron and provide proton and heavy-ion beams over the entire Nuclotron energy range.

## 4.3 Nuclotron 2.1: quasi-frozen spin for deuterons

Adapting the Nuclotron lattice to the measurement of the deuteron EDM involves three problems: lengthening the straight sections, suppressing the dispersion in the straight sections, and keeping the spin direction along the momentum around the ring.

The first problem is solved by raising the maximum field of the bending magnets to 1.8 T and by merging the two adjacent magnets between the quadrupoles into one magnet. At the same time, the dispersion is suppressed by choosing the horizontal phase advance in the arcs. One superperiod of the modernized Nuclotron 2.1 is shown in Fig. 21. The main parameters of Nuclotron 2.1 are: circumference 251 m; betatron tunes Qx=9.78548, Qy=10.6839; momentum compaction factor 0.0134394; slip factor −0.0222388; natural chromaticity −16.0394 (horizontal) and −17.8984 (vertical). In the figure, D denotes a defocusing quadrupole, F a focusing quadrupole, B a dipole magnet and ED an electrostatic deflector. The parameters of all options are summarized in Table 5.

Figure 21. Twiss functions of one superperiod of the modernized Nuclotron 2.1 with two electrostatic deflectors.

With the bending field raised to 1.8 T, the length of the straight section in each superperiod increases from 7.3 to 10.5 m. The third problem, keeping the spin direction relative to the momentum as required for the detection of the deuteron EDM signal, is solved with negative-curvature electrostatic deflectors in each superperiod (Fig. 22).

In the quasi-frozen spin concept, the magnetic and electric fields act in different elements. The spin direction therefore oscillates relative to the direction of motion, since the spin rotates in opposite directions in the magnetic and electrostatic bending elements. The deuteron magnetic anomaly is small, G=−0.143. The spin oscillates around the momentum in each magnetic arc within half of the spin phase advance, Φs=12N 2πγG, and each time it is turned back by −Φs in the electrostatic deflector; here N is the arc superperiodicity. This keeps the average spin direction along the momentum around the whole ring, in accordance with the quasi-frozen spin concept [25].

For the Nuclotron, N=8. Owing to the small value of Φs, the effective EDM signal is reduced only by the factor

J0(Φs)≈1−Φs24,  (20)

i.e., by a few percent at most.

To determine the parameters of the electrostatic deflectors in the quasi-frozen spin mode for deuterons, consider the lattice as consisting of two different parts (Fig. 22). The magnetic arcs bend the beam by ΦB=2π/N+2α each and rotate the spin in the horizontal plane relative to the momentum by ΦsB=νsBΦB. The electrostatic arcs with negative-curvature deflectors bend the beam by ΦE=−2α each and rotate the spin relative to the momentum in the opposite direction by ΦsE=νsEΦE. The quasi-frozen spin condition requires ΦsE=−ΦsB. Since in an electrostatic deflector the spin rotates relative to the momentum many times faster than in a magnetic field, the two types of arcs are related by

νsB2πN+2α=νsE 2α,  α=1νsE/νsB−1 πN=−γ2GG+1 πN,  (21)

where νsE and νsB are given by Eqs. (5) and (6). For given bending magnets and a deflector field EED=130 kV/cm, one obtains νsE/νsB, the parameter α and the radius of curvature of the deflector RED=κ/EED, where κ=p0βc/e is the electric rigidity and p0=γmβc, so that RED=mc2eγβ2EED. The deflector length is then LED=2αRED=−2γ2GG+1πNRED. Figure 22 shows how the electrostatic deflectors are inserted to realize the quasi-frozen spin lattice of the Nuclotron. At the optimal polarimeter energy W=270 MeV [12], α=0.026π and the required length of the electrostatic channel is 7.3 m. The EDM signal relative to the frozen spin case, SEDMQFS/SEDMFS=1−α2/4≈0.998, is reduced by only 0.2%.

The electrostatic arc with negative curvature leaves room for the necessary equipment in the straight section between two magnetic arcs; when the electrostatic deflectors are switched off, the beam passes straight through this section.

Figure 22. Electrostatic insertion compensating the spin rotation in the magnetic arc.

A lattice with twice the number of superperiods, 16, was also considered. By splitting the double bending magnets into individual magnets, the number of FODO cells per superperiod is doubled, and with a 60° phase advance per FODO cell the dispersion is suppressed in every second straight section. In this version α=0.013π, and the EDM signal relative to the frozen spin case is 1−α2/4≈0.9996, i.e., reduced by only 0.04%, which makes this lattice practically equivalent to the frozen spin one. This lattice can also be used for the proton EDM, since the spin rotation angle of protons in the magnetic arcs is only about 30°.

## 4.4 Nuclotron 2.2: quasi-frozen spin with Wien filters for deuterons

In the Nuclotron 2.2 lattice, a small vertical magnetic field of about 100 mT is added to compensate the Lorentz force of the electric field, so that the electrostatic deflectors become straight. With the magnetic field added, the E+B element is a Wien filter. The ring then consists of arcs connected by straight sections with Wien filters, in which crossed electric and magnetic fields keep the particles on a straight path (zero Lorentz force) and compensate the spin rotation accumulated in the magnetic arcs. The total length of the Wien filters is determined by the beam parameters and by the required compensation of the spin rotation in the arcs, whereas their number is chosen for convenient placement around the ring.

With N magnetic arcs in total, each arc bends the particles by ΦarcB=2π/N and rotates the spin in the horizontal plane relative to the momentum by Φsarc=γG ΦarcB. The Wien filters rotate the spin relative to the momentum in the opposite direction. In the electric field Ewf the spin rotation angle is

ΦswfE=−γG+γγ+1β2 ΦwfE,  (22)

where ΦwfE=eEwfm0c2γβ2Lwf is the momentum rotation angle in the electric field of the Wien filter, and in the magnetic field Bwf it is

ΦswfB=(γG+1) ΦwfB,  (23)

where ΦwfB=eBwfm0cγβLwf is the momentum rotation angle in the magnetic field of a Wien filter of length Lwf.

Since the Lorentz force in the Wien filter vanishes, the angles must be equal, ΦwfE=ΦwfB, and they can be expressed through the electric field Ewf and the length Lwf of the filters installed in all N straight sections. In the frozen spin option, the total spin precession frequency relative to the momentum in the combined E+B bending element is zero. The quasi-frozen spin option instead requires the total spin rotation angle per turn in the E and B fields to vanish. The QFS condition therefore reads

(γG+1) ΦwfB−γG+γγ+1β2 ΦwfE=γG 2πN,  (24)

which gives the parameters of one Wien filter:

LwfEwf=2πN GG+1 m0c2e β2γ3,  Bwf=−Ewfcβ,  (25)

where Lwf is the length of one Wien filter. The total length of all Wien filters, LwfΣEwf=2πGG+1m0c2eβ2γ3, depends only on the particle species and energy.

Equations (21) and (25), derived for electrostatic deflectors and for Wien filters, show that the type of the compensating element does not matter. With the magnetic arc unchanged, the Wien filters are shorter by the total length of the kickers, since a Wien filter combines the functions of an electrostatic deflector and a kicker in one element.

Figure 23 shows the Twiss functions of one superperiod of the first version of the modernized lattice, Nuclotron 2.2.1. Two Wien filters installed in the central straight sections of the superperiod restore the spin direction after each magnetic half-arc. The spin deviation angle from the momentum, α=γG 2π/N, characterizes how much the quasi-frozen spin lattice differs from the frozen spin one. In this lattice, the number of arcs that determine the spin deviation equals the number of superperiods, N=8.

Figure 23. Twiss functions of one superperiod of the modernized Nuclotron 2.2.1 with two Wien filters.

For a maximum electric field of 130 kV/cm, the magnetic field is below 80 mT. Such a low magnetic field simplifies the Wien filter design: a permanent magnet or an air-core coil can be used. For these fields and a deuteron energy of 270 MeV, the total length of the Wien filters in all straight sections is about 52 m, and the length of one Wien filter is 3.25 m. At 270 MeV (135 MeV per nucleon), the maximum spin deviation is α∼±3∘ over one arc, which is a good approximation to a regular frozen spin lattice.

In the next version, Nuclotron 2.2.2, the Wien filters are distributed over four straight sections of the superperiod (Fig. 24). Each arc of the superperiod is thus split into two arcs, and with respect to the bending magnets the lattice has the superperiodicity N=16, while the overall superperiodicity of the lattice with all elements remains 8. This solves two problems. First, the ring becomes a 16-sided polygon, i.e., closer to a circle, and fits the existing tunnel better. Second, with a bending magnet length of 1.77 m and a Wien filter length of 1.65 m, the spin deviation is α∼±1.5∘, half that of version 2.2.1. This brings the properties of the quasi-frozen spin lattice much closer to those of the frozen spin lattice.

Figure 24. Twiss functions of one superperiod of the modernized Nuclotron 2.2.2 with four Wien filters.

To increase the energy, the modification Nuclotron 2.2.3 with fewer empty drift spaces and longer magnets was considered (Fig. 25). In this lattice, the maximum energy is 3.5 GeV per nucleon for deuterons and 7.8 GeV for protons, which is much closer to the present Nuclotron.

Figure 25. Twiss functions of one superperiod of the modernized Nuclotron 2.2.3 with four Wien filters.

Table 5. Main parameters of the Nuclotron modifications.

| Nuclotron lattice | Magnet length, m | Number of magnets | Total magnet length, m | Bmax, T | BmaxR, T⋅m | Wdmax, GeV | Wdmax, GeV/u | Wpmax, GeV |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Present | 1.44 | 96 | 138.24 | 1.8 | 39.6 | 10.14 | 5.07 | 10.97 |
| 2.1 | 2.35 | 48 | 112.77 | 1.8 | 32.2 | 8 | 4 | 8.79 |
| 2.2.1 | 1.78 | 48 | 85.3 | 1.8 | 24.4 | 5.7 | 2.85 | 6.47 |
| 2.2.2 | 1.78 | 48 | 85.44 | 1.8 | 24.5 | 5.7 | 2.85 | 6.47 |
| 2.2.3 | 2.10 | 48 | 101 | 1.8 | 28.93 | 7.0 | 3.5 | 7.8 |

## 4.5 Nuclotron 2.2.2: quasi-frozen spin with Wien filters for protons

With a magnetic superperiodicity of 16, the lattice can also be used for protons at a reduced energy. The energy is then limited mainly by the free space for the Wien filters, which is 52 m in this lattice. The maximum proton energy follows from Eq. (25):

γ(γ2−1)=Lwf eEwfmc2 N2π Gp+1Gp,  (26)

where LwfN=52 m and Gp=1.793 is the proton magnetic anomaly. Solving for γ gives γ=1.08047, i.e., Wp≈80 MeV. At this energy, the spin deviation in a magnetic arc is determined by γGp=1.93711, and the spin deviation angle is α=±12γGp16, which amounts to α=3.5∘.

[Note: Check: with the factor 2π of Eq. (25), α=±12γGp2π16≈22∘.]

The same lattice as for deuterons can be used for protons, but at a lower energy and with the Wien filters rotated by 180°, since the magnetic anomalies of the two particles have opposite signs. At a proton energy of 80 MeV the lattice thus behaves practically as a frozen spin lattice. Figure 26 shows the p–C analyzing power in terms of the “average analyzing power” taken from Ref. [26].

Figure 26. Average p–C analyzing power Ac for θlab=5∘–20° at discrete energies (points) and its continuous energy dependence (solid line). The data are from SIN (△), TRIUMF (×) and LAMPF (•) [26].

Figure 26 shows that the analyzing power near 100 MeV is about 0.2, i.e., 0.35 of its maximum value Ac=0.53 at 230 MeV. This makes proton EDM studies at this energy in the Nuclotron 2.2.2 lattice feasible in parallel with full-scale deuteron measurements.

Galactic axions can be searched for with the same equipment as the EDM. Varying the spin precession frequency at a fixed beam energy turns the booster with Wien filters into a broadband antenna that searches for axions through a resonant full or partial spin flip in the oscillating pseudomagnetic field of the axion halo of our Galaxy [27]. The growth of the horizontal polarization induced by axions is extracted by a Fourier analysis of the oscillating vertical asymmetry in the internal polarimeter [28].

## 4.6 Maximum energy of heavy ions and protons in Nuclotron 2.2.2

The calculations show that deuteron and proton EDM experiments in the Nuclotron 2.2.2 lattice require free space for Wien filters with a total length of 52 m. With the tunnel circumference fixed at 251 m, this is possible either by raising the field of the bending magnets, which is already set to 1.8 T, or by reducing their total length. Assuming that the bending field does not exceed 1.8 T, the second option was chosen, which inevitably lowers the maximum particle energy in the Nuclotron.

## 4.7 Features of the modernized Nuclotron lattice

The lattice of the modernized Nuclotron was designed with several goals. First, the superperiod has a drift without bending magnets in its central cell, where the dispersion function reaches its maximum and the orbit curvature is zero (ρ→∞). This makes it possible to raise the transition energy, since the momentum compaction factor is

α=1C∮D(s)ρ(s) ds.  (27)

Second, the number of superperiods s=8 is only slightly larger than the horizontal betatron tune νx, which makes the momentum compaction factor easy to adjust, as in the resonant lattices with a low or negative momentum compaction factor [29–33]:

α=νx3Rk=0∞ak2νx2−(ks)2,  (28)

where ak are the harmonics of the dispersion-generating perturbation. Third, the central drift simplifies the placement of the three sextupole families needed to control the betatron and spin chromaticities, whereas the side drifts are convenient for the Wien filters. Finally, the maximum of the dispersion at the center of the superperiod coincides with the minimum of the horizontal beta function βx.

## 4.8 Summary

If the Nuclotron is used as the booster of the collider, its final energy can be lowered to 2–3 GeV without a significant degradation of the NICA complex, because all physics experiments with polarized beams are transferred to the collider. This choice has a further advantage: it shortens the total length of the bending magnets, e.g., by reducing each double bending magnet of the Nuclotron to one section, and frees about 70 m of the orbit for additional equipment, in particular for polarization-preserving solenoids. It also provides room for EDM and axion search equipment directly in the Nuclotron.

The material of this section has been reported in [2,11,34–37].

# 5 Specialized frozen spin rings at JINR

This section presents two specialized frozen spin lattices designed exclusively for the search for the electric dipole moments of the proton and the deuteron: an all-electrostatic ring for protons and a ring with combined E+B deflectors for deuterons.

## 5.1 Frozen spin option for the proton beam

The frozen spin lattice for a proton beam is the simplest one in terms of parameter choice and is uniquely defined. It consists of electrostatic deflectors whose total length Ltotal is determined by the beam energy and the maximum permissible electrostatic field strength:

Ltotal=2π mc2eE γβ2.  (29)

For an energy of 233 MeV and a maximum field of E=120 kV/cm, the total length of all electrostatic deflectors is Ltotal=219.55 m.

With 50 deflectors placed around the ring, the length of a single deflector is Ldef=4.39 m. The 50 deflectors are divided into five groups, each consisting of five FODO cells and containing ten deflectors (Fig. 27).

Figure 27. Twiss functions of one superperiod of the electrostatic proton ring.

Figure 28 shows the whole ring, which consists of five superperiods. Each superperiod comprises an achromat with drift spaces at both ends, so that the phase advance per superperiod is not an integer multiple of 2π. The ring circumference is 419.1 m. The layout of the electrostatic ring is shown in Fig. 29; it closely resembles a circle.

Figure 28. Twiss functions along the whole electrostatic proton ring.

Figure 29. Layout of the electrostatic proton ring.

Table 6 summarizes the ring parameters required to estimate its dimensions and cost.

Table 6. Parameters of the frozen spin electrostatic ring for the proton EDM search.

| Parameter | Value |
| --- | --- |
| Circumference, m | 419.1 |
| Symmetry order (number of arcs and straight sections) | 5 |
| Injection energy, MeV | 20 |
| Proton magnetic anomaly G | 1.793 |
| Final energy, MeV | 233.8 |
| EDM experiment energy, MeV | 233.8 |
| Initial and final relative velocity and Lorentz factor | β=0.2, γ=1.0213; β=0.6, γ=1.248 |
| Revolution frequency at the beginning and end of the cycle, MHz | 0.143–0.429 |
| Harmonic number | 20 |
| RF frequency at the beginning and end of the cycle, MHz | 2.8–8.6 |
| Arcs |  |
| Arc length, m | 70.1 |
| Arc superperiodicity | regular lattice |
| Arc lattice | 5 FODO cells |
| Number of electrostatic deflectors per arc | 10 |
| Deflector length, m | 4.39 |
| Deflector type | cylindrical |
| Maximum electric field of the deflector, MV/m | 12 |
| Number of quadrupoles per arc | 9 + 1 (two end halves) |
| Quadrupole type | electrostatic |
| Quadrupole families | one focusing (LF), one defocusing (LD) |
| Maximum quadrupole gradient in the arcs, kV/cm​2 | LF =6.78, LD =−6.18 |
| Quadrupole length, m | 1.0 |
| Number of chromatic sextupoles per arc | 10 |
| Sextupole type | electric or magnetic |
| Sextupole strengths | to be determined |
| Betatron tunes νx/νy | 6.12/4.72 (preliminary) |
| Natural chromaticity ξx/ξy | to be determined |
| Transition energy, GeV | 3.1 |
| Maximum βx/βy, m | 27.9/34.8 |
| Maximum horizontal dispersion, m | 10.8 |
| Straight sections between arcs |  |
| Length of straight sections, m | 5×13.5 |
| Lattice of straight sections | DOFFOD |
| Quadrupole gradients, T/m | QF =7.93, QD =−7.11 |
| Drift length between adjacent quadrupoles, m | 6.0 |

As a second option, a racetrack ring with two arcs and dispersion suppressors at both ends of each arc was considered (Fig. 30). One of the two options will be adopted in the course of further lattice development, which will take spin decoherence into account [38].

Figure 30. Arc with dispersion suppressors at both ends (racetrack option of the proton ring).

## 5.2 Frozen spin option for the deuteron beam

The frozen spin lattice for a deuteron beam differs from the proton one in the composition of the equipment and in the elements used. Here the bending deflector with E+B fields consists of electrostatic plates generating a horizontal electric field, inserted into the gap of a dipole magnet with a vertical magnetic field (Fig. 31).

(a); (b).

Figure 31. Electrostatic deflector (a) and vertical-field dipole magnet with the electrostatic deflector inserted into its gap (b).

The condition of stable motion on an orbit of radius R follows from the balance of the Lorentz and centrifugal forces:

R=γm0v2eE+cβ×B,  (30)

from which the momentum precession frequency in the combined fields is

Ωp=eEm0γβc+e cβ×Bm0γβc.  (31)

Subtracting Ωp from Ωmdm gives the spin precession frequency relative to the momentum:

Ωmdmp=em0γγGB+1γ2−1−Gβ×Ec.  (32)

A frozen spin lattice for EDM studies has to satisfy two conditions. The first relates the external fields to the orbit radius, Eq. (30) (see also [7]). The second requires the spin precession frequency relative to the direction of motion to vanish, Ωmdmp=0, i.e., γGB=−1γ2−1−Gβ×Ec. This fixes the relation between the fields,

E=−GBcγ2β2β1−γ2β2G,  (33)

and the radius of curvature of the deflector,

R=γm0v2eE1−1Gγ2.  (34)

For an energy of 270 MeV, the radius of curvature of the deflector is R=9.207 m, and the total length of all deflectors is LΣ=2πR=57.85 m. With 32 deflectors in total, the length of a single deflector is Ldef=LΣ/32=1.808 m.

Figure 32 shows the vector diagram that defines the relative directions of E and B. The determining factor is the positive vertical direction of the magnetic field, which provides the main compensation of the centrifugal force. Since the deuteron magnetic anomaly is negative, G=−0.143, the axis around which S precesses points downwards. A zero precession frequency then requires the vector β×E to point upwards, i.e., the electric field must be directed as shown in Fig. 32.

Figure 32. Vector diagram of the fields acting on a polarized deuteron.

Based on these relations, a frozen spin ring lattice was developed. Figure 33 shows the lattice of one arc for the deuteron beam. The arc consists of two successive achromats with zero dispersion at its middle and at its ends and comprises eight FODO cells.

Figure 33. Twiss functions along an arc of the frozen spin deuteron ring.

As in the electrostatic proton ring, the dispersion modulation introduced in the deuteron lattice provides suitable locations for sextupoles suppressing the spin chromaticity. Figure 34 shows the Twiss functions of the whole ring with a circumference of 146 m; its layout is shown in Fig. 35. The main parameters of the synchrotron for the deuteron EDM search at 270 MeV per nucleon are listed in Table 7.

Figure 34. Twiss functions along the frozen spin deuteron ring.

Figure 35. Layout and dimensions of the frozen spin deuteron ring.

Table 7. Parameters of the frozen spin ring with E+B deflectors for the deuteron EDM search.

| Parameter | Value |
| --- | --- |
| Circumference, m | 145.85 |
| Symmetry order (number of arcs and straight sections) | 2 |
| Injection energy, MeV/u | 40 |
| Final energy, MeV/u | 270.0 |
| EDM experiment energy, MeV/u | 270.0 |
| Deuteron magnetic anomaly G | −0.143 |
| Initial and final relative velocity and Lorentz factor | β=0.2, γ=1.0213; β=0.485, γ=1.143 |
| Revolution frequency at the beginning and end of the cycle, MHz | 0.143–0.99 |
| Harmonic number h | 50 |
| RF frequency at the beginning and end of the cycle, MHz | 7.15–49.9 |
| Length of the quarter-wave RF cavity, m | 1.502 |
| Momentum acceptance of the separatrix at maximum energy | 1.31×10−3 |
| Relative momentum spread in the bunch | 5×10−4 |
| Arcs |  |
| Arc length, m | 51.27 |
| Arc superperiodicity | 2 |
| Arc lattice | 4 + 4 FODO cells |
| Number of E+B deflectors per arc | 16 |
| Deflector length, m | 1.80 |
| Deflector type | combined magnetic and electric |
| Maximum electric field of the deflector, MV/m | 12 |
| Number of quadrupoles per arc | 15 + 1 (two end halves) |
| Quadrupole type | magnetic multipole |
| Quadrupole families | one focusing (QF), one defocusing (QD) |
| Maximum quadrupole gradient in the arcs, kG/cm | QF =1.359, QD =−1.00 |
| Quadrupole length, m | 0.1 |
| Number of chromatic sextupoles per arc | 16 |
| Sextupole type | magnetic |
| Sextupole strengths, kG/cm​2 | SF =0.12, SD =0.15 |
| Betatron tunes νx/νy | 4.70418/2.6140 (preliminary) |
| Natural chromaticity ξx/ξy | −4.91629/−3.99402 |
| Transition energy, GeV | 5.3 |
| Maximum βx/βy, m | 15.1/16.5 |
| Maximum horizontal dispersion, m | 2.5 |
| Straight sections between arcs |  |
| Length of straight sections, m | 2×23.3 |
| Lattice of straight sections | 3×FODO |
| Quadrupole gradients, kG/cm | QF =0.734, QD =−0.782 |
| Free drift spaces between adjacent quadrupoles, m | 12×3.2 |

## 5.3 Summary

Two specialized frozen spin lattices have been developed for the search for the proton and deuteron electric dipole moments. These results form the basis for the physical design of the experimental facilities and their cost estimate. In contrast to the quasi-frozen spin options of Sections 3 and 4, these rings are intended exclusively for experiments on the violation of fundamental symmetries.

The material of this section has been reported in [39,40].

# 6 Transition energy crossing for polarized protons

[Note: Draft: this chapter is incomplete and will be extended.]

Collider experiments impose requirements on the luminosity, which, among other things, limit the longitudinal emittance of the final bunch. During acceleration, the beam must both cross the transition energy and be split into 22 bunches by RF gymnastics without blowing up the longitudinal phase volume.

A possible transition jump scheme for NICA has been considered. Its characteristic values are the jump amplitude Δγtr=0.09 and the rate of change of the transition energy dγtr/dt=8.5 s−1. With a harmonic RF system, the proposed jump has little effect on the longitudinal dynamics, since the jump is small and slow compared with the acceleration rate. With a barrier-bucket RF system, the jump amplitude is limited by the microwave instability threshold for the number of particles in a uniform bunch. This limit does not allow reaching 1×1012 particles in the final bunch required for the maximum luminosity.

The material of this section has been reported in [41,42].

# 7 Spinor technique for spin-orbit tracking

To be written.

# 8 Electrostatic elements

To be written.

# 9 MDM systematics and tensor polarizability

To be written.

# 10 Spin precession in the axion search mode

Peccei–Quinn axions, proposed as a solution of the strong CP problem, are among the most likely dark matter candidates. A storage ring can be used as an antenna for the search for the galactic axion [27].

Current searches for resonant spin rotation in storage rings use the method developed by the JEDI collaboration, in which the growth of the vertical polarization out of the horizontal plane indicates the presence of the axion field [27]. Based on an analytical treatment of the effect of the spin coherence time on the frequency scan for the axion signal, an alternative scheme has been proposed in which an initially vertical spin is rotated into the horizontal plane [27]. This scheme is free from the ambiguity of the axion field phase, requires no RF spin flippers, and can be readily implemented with both deuterons and protons stored in the Nuclotron and NICA rings.

Consider the proposed experiment on the example of the deuteron EDM search with polarized beams. It is based on bypasses with Wien filters installed on them (Section 3); the QFS lattice of NICA with bypasses is therefore of particular interest. The EDM and axion experiments differ in that the axion search does not require the frozen spin condition.

The method relies on the interaction of the axion with the EDM of the particles in the bunch. This interaction modifies the spin precession described by the T-BMT equation (1) and increases the vertical spin component. Since the axion mass is unknown, it has to be found by scanning the spin precession frequency, which is done by varying the fields of the Wien filters.

In the bending arcs of the NICA collider, the particle momentum rotates by ΦarcB=π, while the deuteron spin (γG<0) rotates in the opposite direction in the horizontal plane relative to the momentum by the angle Φsarc of Eq. (7) (Fig. 36). In each of the two straight sections, the Wien filters rotate the spin back as described by Eqs. (8)–(10). For a maximum electric field of 120 kV/cm, the required magnetic field is below 80 mT.

Figure 36. Vector diagram of the spin transformation for the deuteron.

In the axion search mode, the QFS condition (9) is modified to

(γG+1) ΦssB−γG+γγ+1β2 ΦssE=ΔΦ+πγG,  (35)

where ΔΦ is the detuning phase with respect to the QFS case ΔΦ=0. By changing the energy and the Wien filter fields Bss and Ess, one changes the frequency of the spin precession around the vertical axis,

fspin=ΔΦ cβCring,  (36)

where Cring is the ring circumference, cβ is the beam velocity and ΔΦ is the detuning phase defined by Eq. (35). For example, at ΔΦ=1 rad the spin precession frequency around the vertical axis is 270 kHz, i.e., the axion search range is fspin=0–270 kHz.

[Note: Eq. (36) gives an angular frequency (rad/s); the frequency in Hz is fspin/2π.]

For a proton beam, the QFS option cannot be realized in NICA: the proton magnetic anomaly Gp=1.793 is an order of magnitude larger, and the spin rotates by more than 360° in each arc, so that the accumulated EDM signal practically vanishes. For the axion search, however, the idea remains valid, in a different range of fspin.

The material of this section has been reported in [18,27,43].

# 11 Conclusion

Three lattice options for a storage-ring search for the electric dipole moments of light nuclei at JINR have been presented.

The NICA collider can be used as a storage ring for EDM experiments by adding bypass channels parallel to the original straight sections. Wien filters installed in the bypasses compensate the MDM spin rotation in the arcs, while the arcs remain unchanged, so that NICA keeps its collider functions. The most realistic bypass, with a fully regular straight section, satisfies all optical requirements, and spin-orbit tracking confirms that the quasi-frozen spin method can be realized in NICA. The simultaneous control of betatron and spin chromaticity is under study, and the same lattice allows NICA to be used as an axion antenna.

The Nuclotron can be modernized for deuteron and proton EDM experiments in the quasi-frozen spin mode with electrostatic deflectors or Wien filters. If the Nuclotron serves as the collider booster, its final energy can be lowered to 2–3 GeV, which frees about 70 m of the orbit for polarization-preserving solenoids and EDM and axion equipment.

Finally, two specialized frozen spin rings have been designed: a purely electrostatic proton ring with a circumference of 419.1 m and a deuteron ring with combined E+B deflectors with a circumference of 145.85 m. These lattices form the basis for the physical design and cost estimate of facilities intended for the search for the violation of fundamental symmetries.

# Acknowledgments

This work was supported by the Russian Science Foundation, grant No. 25-72-30005 (https://rscf.ru/project/25-72-30005/).

# References

[1] 	D. Anastassopoulos et al., Search for a permanent electric dipole moment of the deuteron nucleus at the 10−29 e⋅cm level, (2008).

[2] 	Yu. Senichev, A. Aksentyev, S. Kolokolchikov, A. Melnikov, N. Nikolaev, V. Ladygin, and E. Syresin, NICA Facilities for the Search for EDM Light Nuclei, Phys. Atom. Nucl. 87, 436 (2024).

[3] 	G. Guidoboni et al., How to Reach a Thousand-Second in-Plane Polarization Lifetime with 0.97-GeV/c Deuterons in a Storage Ring, Phys. Rev. Lett. 117, 054801 (2016).

[4] 	V. Bargmann, L. Michel, and V. L. Telegdi, Precession of the polarization of particles moving in a homogeneous electromagnetic field, Phys. Rev. Lett. 2, 435 (1959).

[5] 	T. Fukuyama and A. J. Silenko, Derivation of Generalized Thomas-Bargmann-Michel-Telegdi Equation for a Particle with Electric Dipole Moment, Int. J. Mod. Phys. A 28, 1350147 (2013).

[6] 	Y. Senichev, S. Andrianov, M. Berz, S. Chekmenev, A. Ivanov, and E. Valetov, Investigation of Lattice for Deuteron EDM Ring, in 12th International Computational Accelerator Physics Conference (2016), pp. 17–19.

[7] 	F. J. M. Farley and Y. K. Semertzidis, The 47 years of muon g-2, Prog. Part. Nucl. Phys. 52, 1 (2004).

[8] 	Yu. Senichev, A. Aksentyev, S. Kolokolchikov, and A. Melnikov, Features of the Spin-orbital Dynamics of a Polarized Beam in Electrostatic and Magnetostatic Fields in the Study of the Electric Dipole Moment of light Nuclei, Phys. Atom. Nucl. 87, 1467 (2024).

[9] 	A. A. Melnikov, Yu. V. Senichev, A. E. Aksentyev, and S. D. Kolokolchikov, Investigation of the Quasi-Frozen Spin Lattice for the EDM Search, Phys. Atom. Nucl. 88, 1999 (2025).

[10] 	A. Melnikov, A. Aksentyev, N. Nikolaev, S. Kolokolchikov, and Y. Senichev, Quasi-frozen spin concept to search for the electric dipole moment of the proton and deuteron, JACoW IPAC2025, TUPS033 (2025).

[11] 	Y. Senichev, A. Melnikov, A. Aksentyev, E. Syresin, P. Palamarchuka, S. Kolokolchikov, and V. Ladygin, Wien filter method for the ”Quasi-frozen” spin lattice, JACoW IPAC2025, TUPS032 (2025).

[12] 	P. K. Kurilkin et al., The 270 MeV deuteron beam polarimeter at the Nuclotron Internal Target Station, Nucl. Instrum. Meth. A 642, 45 (2011).

[13] 	V. Lebedev, OptiM: computer code for linear and non-linear optics calculations, (n.d.).

[14] 	S. Guiducci, Chromaticity, in CERN Accelerator School: Course on General Accelerator Physics (1992), pp. 191–206.

[15] 	K. Makino and M. Berz, COSY INFINITY Version 9, Nucl. Instrum. Meth. A 558, 346 (2006).

[16] 	A. A. Melnikov, Yu. V. Senichev, A. E. Aksentyev, and S. D. Kolokolchikov, Nature of Spin Decoherence of a Polarized Beam of Light Nuclei in a Storage Ring for EDM Search, JETP Lett. 118, 710 (2023).

[17] 	A. Melnikov, A. Aksentyev, Y. Senichev, and E. Syresin, Investigation of polarized proton spin coherence time at storage rings, JACoW IPAC2022, 1832 (2022).

[18] 	Y. Senichev, A. Aksentyev, S. Kolokolchikov, A. Melnikov, V. Ladygin, E. Syresin, and N. Nikolaev, Quasi-frozen spin concept of magneto-optical structure of NICA adapted to study the electric dipole moment of the deuteron and to search for the axion, JACoW IPAC2022, MOPOTK024 (2022).

[19] 	S. Kolokolchikov, A. Aksentyev, A. Melnikov, Y. Senichev, E. Syresin, and V. Ladygin, ByPass optics design in NICA storage ring for experiment with polarized beams for EDM search, JACoW IPAC2023, MOPA072 (2023).

[20] 	S. Kolokolchikov, A. Aksentyev, A. Melnikov, and Y. Senichev, Spin coherence and betatron chromaticity of deuteron beam in NICA storage ring, JACoW IPAC2023, MOPA070 (2023).

[21] 	S. Kolokolchikov et al., Spin coherence and betatron chromaticity of a deuteron beam in the quasi-frozen spin mode, in Proc. LaPlas-2023 (2023), p. 290.

[22] 	A. E. Aksentev, A. A. Melnikov, Y. V. Senichev, S. D. Kolokolchikov, V. P. Ladygin, E. M. Syresin, and A. V. Butenko, A feasibility study into the quasi-frozen spin regime of operation of the NICA storage ring, JACoW IPAC2023, MOPA071 (2023).

[23] 	A. Melnikov, A. Aksentyev, Yu. Senichev, and S. Kolokolchikov, Quasi-Frozen Spin Concept at NICA for EDM Search and Its Matrix Analysis, Phys. Atom. Nucl. 87, 447 (2024).

[24] 	A. E. Aksentev, A. A. Melnikov, Yu. V. Senichev, and S. D. Kolokolchikov, Spin-Flipping When Approaching the Resonant Energy in the Perturbed NICA Collider Facility with Bypasses, Phys. Atom. Nucl. 87, 1363 (2024).

[25] 	Y. Senichev and A. Aksentyev, Features of Quasi-frozen Spin Concept of Storage Ring for Searching for Electric Dipole Moment of Deuteron, J. Phys. Conf. Ser. 1686, 012068 (2020).

[26] 	M. W. Mcnaughton et al., THE P C ANALYZING POWER BETWEEN 100-MEV AND 750-MEV, Nucl. Instrum. Meth. A 241, 435 (1985).

[27] 	N. N. Nikolaev, Spin of protons in NICA and PTR storage rings as an axion antenna, Pisma Zh. Eksp. Teor. Fiz. 115, 683 (2022).

[28] 	D. Eversmann et al., New method for a continuous determination of the spin tune in storage rings and implications for precision experiments, Phys. Rev. Lett. 115, 094801 (2015).

[29] 	Y. Senichev, A Resonant lattice for a synchrotron with a low or negative momentum compaction factor, KEK Preprint 97-40, (1997).

[30] 	Y. Ishi, S. Machida, Y. Mori, and S. Shibuya, Lattice design of JHF synchrotrons, Conf. Proc. C 9803233, 378 (1998).

[31] 	Y. Papaphilippou, W. Bartmann, H. Bartosik, M. Benedikt, B. Goddard, A. Lachaize, and Y. Senichev, Comparison of PS2 Lattices with Different Geometries, Conf. Proc. C 100523, THPE021 (2010).

[32] 	Yu. Senichev, The lattice with imaginary gamma-transition for the CERN proton synchrotron PS2, in CARE-HHH-APD Workshop on Finalizing the Roadmap for the Upgrade of the CERN & GSI Accelerator Complex (BEAM’07) (2008), p. 171.

[33] 	R. Toelle et al., HESR at FAIR: Status of technical planning, Conf. Proc. C 070625, 1442 (2007).

[34] 	Yu. V. Senichev, A. E. Aksentyev, S. D. Kolokolchikov, A. A. Melnikov, V. P. Ladygin, and E. M. Syresin, Consideration of an Adapted Nuclotron Structure for Searching for the Electric Dipole Moment of Light Nuclei, Phys. Atom. Nucl. 86, 2434 (2023).

[35] 	Yu. V. Senichev, S. D. Kolokolchikov, A. E. Aksentyev, A. A. Melnikov, N. N. Nikolaev, V. P. Ladygin, and E. M. Syresin, Modernization of the Magneto-Optical Structure of the Nuclotron to Study the Possibility of Measuring the Electric Dipole Moment of Light Nuclei with Preservation of Functions of the Booster of Polarized Beams for the NICA Collider, Phys. Atom. Nucl. 88, 2025 (2025).

[36] 	Yu. V. Senichev, S. D. Kolokolchikov, A. E. Aksentyev, A. A. Melnikov, N. N. Nikolaev, V. P. Ladygin, and E. M. Syresin, Highly Superperiodic Structure of the Nuclotron with Quasi-Frozen Spin for Studying the Electric Dipole Moment of Light Nuclei, Phys. Part. Nucl. Lett. 23, 206 (2026).

[37] 	S. Kolokolchikov, A. Melnikov, A. Aksentyev, E. Syresin, P. Palamarchuka, V. Ladygin, and Y. Senichev, Electrostatic deflector Nuclotron modernization for EDM experiment, JACoW IPAC2025, TUPS031 (2025).

[38] 	Y. Senichev, R. Maier, D. Zyuzin, and N. Kulabukhova, Spin Tune Decoherence Effects in Electro- and Magnetostatic Structures, in 4th International Particle Accelerator Conference (2013), pp. 2579–2581.

[39] 	P. I. Palamarchuka, S. D. Kolokolchikov, A. E. Aksentyev, A. A. Melnikov, and Yu. V. Senichev, Development of a Conceptual Storage Ring Structure for the Study of the Electric Dipole Moment of Light Nuclei, Phys. Part. Nucl. Lett. 23, 365 (2026).

[40] 	S. D. Kolokolchikov, Y. V. Senichev, A. E. Aksentyev, A. A. Melnikov, and P. I. Palamarchuka, Quasi-frozen spin for both deuteron and proton beams in a periodic EDM storage ring lattice, Nucl. Sci. Tech. 37, 148 (2026).

[41] 	S. Kolokolchikov, Yu. Senichev, A. Aksentiev, A. Melnikov, E. Syresin, and V. Ladygin, Transition Energy Crossing of Polarized Proton Beam at NICA, Phys. Atom. Nucl. 87, 212 (2024).

[42] 	S. Kolokolchikov, Y. Senichev, A. Melnikov, and E. Syresin, Acceleration and crossing of transition energy investigation using an RF structure of the barrier bucket type in the NICA accelerator complex, JACoW IPAC2022, WEPOPT004 (2022).

[43] 	A. Aksentev, A. Melnikov, Yu. Senichev, S. Kolokolchikov, N. Nikolaev, and P. Palamarchuka, First Look into the Potential of the Proton Spin in a Quasi-Frozen Spin Lattice as an Axion Detector, Phys. Part. Nucl. Lett. 23, 360 (2026).